CONFIGURE, ANALYZE AND EVALUATE A SIMULATED

COMPUTER NETWORK USING IP ADDRESSING, ROUTING,

PROTOCOLS AND TOPOLOGIES

A Network Design & Simulation Report

Submitted by

S. LOKESH

Register No: 192572151

Course: Computer Networks (CSA0707)

SDG Mapping: SDG 9 – Industry, Innovation and Infrastructure

# Table of Contents

1. Problem Understanding & Requirements Formulation3

2. IP Addressing, Subnetting (VLSM) & Routing3

3. Topology Design & Device Selection4

4. Simulation Tool Usage & Validation5

5. Protocol Analysis & Comparison (TCP, UDP, HTTP, DNS)5

6. Performance Evaluation & Engineering Analysis6

7. Broader Considerations & Professional Responsibility6

8. Pseudocode Deliverable7

9. GitHub Upload8

10. Reflection (Template)8

# 1. Problem Understanding & Requirements Formulation

The organization being modelled is a medium-sized institution with three functional departments – Administration, Academic/Operations, and Accounts – requiring a converged data network of at least 30 end devices. The network must provide:

- A hierarchical IPv4 addressing scheme derived through VLSM so that each department receives an appropriately sized subnet with minimal address wastage.

- Reliable inter-department routing across at least three network segments, with defined default gateways at every subnet.

- A choice of physical topologies (star, mesh, bus and hybrid) matched to the reliability, cost and cabling constraints of each part of the network.

- Verified end-to-end connectivity for core services – HTTP (intranet/web), DNS (name resolution) and file/data transfer over TCP and UDP.

- A repeatable way to compare TCP, UDP, HTTP and DNS behaviour under normal load, increased load, and link congestion/failure, using measurable performance parameters (latency, throughput, packet loss, delay).

Departmental host and service requirements used for sizing the address plan:

| Department / Segment | Role | Estimated Hosts | Key Services Used |
| --- | --- | --- | --- |
| Academic / Operations | Faculty PCs, labs, operations staff | 15 | HTTP intranet, DNS, file transfer, printing |
| Administration | Admin staff, management PCs | 10 | HTTP, DNS, email/file transfer |
| Accounts | Finance/accounts workstations | 8 | HTTP, DNS, database/file transfer |
| Server Farm | Web, DNS, file server(s) | 5 | Hosts HTTP, DNS, FTP services for all departments |

# 2. IP Addressing, Subnetting (VLSM) & Routing

Base address block: 192.168.10.0/24. Subnets are allocated using VLSM, largest requirement first, to minimise wasted addresses. Design goal: each department gets its own broadcast domain and gateway, and inter-router links use minimal /30 subnets.

## 2.1 VLSM Subnet Allocation

| Segment | Hosts Needed | Subnet (CIDR) | Usable Range | Broadcast | Gateway (Router) |
| --- | --- | --- | --- | --- | --- |
| Academic/Operations LAN | 15 | 192.168.10.0/27 | .1 – .30 | .31 | 192.168.10.1 |
| Administration LAN | 10 | 192.168.10.32/28 | .33 – .46 | .47 | 192.168.10.33 |
| Accounts LAN | 8 | 192.168.10.48/28 | .49 – .62 | .63 | 192.168.10.49 |
| Server Farm | 5 | 192.168.10.64/29 | .65 – .70 | .71 | 192.168.10.65 |
| Core–Academic Link | 2 | 192.168.10.72/30 | .73 – .74 | .75 | – |
| Core–Admin Link | 2 | 192.168.10.76/30 | .77 – .78 | .79 | – |
| Core–Accounts Link | 2 | 192.168.10.80/30 | .81 – .82 | .83 | – |
| Admin–Academic Mesh Link | 2 | 192.168.10.84/30 | .85 – .86 | .87 | – |
| Academic–Accounts Mesh Link | 2 | 192.168.10.88/30 | .89 – .90 | .91 | – |
| Accounts–Admin Mesh Link | 2 | 192.168.10.92/30 | .93 – .94 | .95 | – |

Four routers are used: a Core router (R-CORE) and one edge router per department (R-ADMIN, R-ACAD, R-ACCT). Point-to-point /30 links connect R-CORE to every department router (star at the core), and additional /30 links directly connect the three department routers to each other, forming a partial mesh at the routing layer for redundancy – this is the hybrid design referenced in Section 3.

## 2.2 Static Routing Table (representative – R-CORE)

| Destination Network | Subnet Mask | Next Hop | Exit Interface |
| --- | --- | --- | --- |
| 192.168.10.0 | /27 | 192.168.10.74 (R-ACAD) | Se0/0/0 |
| 192.168.10.32 | /28 | 192.168.10.78 (R-ADMIN) | Se0/0/1 |
| 192.168.10.48 | /28 | 192.168.10.82 (R-ACCT) | Se0/0/2 |
| 192.168.10.64 | /29 | Directly connected | Gig0/0 |

Each department router additionally carries a default route (0.0.0.0/0) pointing to its R-CORE-facing interface, and a static route toward each of the other two departments so that the redundant mesh links are used as automatic backup paths if the core link fails. For a production-scale deployment this static plan can be replaced by a dynamic protocol (e.g. single-area OSPF) without changing the address plan – this is discussed as a scalability point in Section 7.

# 3. Topology Design & Device Selection

All four required topologies are demonstrated, each in the part of the network where it is technically and economically appropriate.

| Topology | Where Used | Devices / Media | Cabling Standard | Justification |
| --- | --- | --- | --- | --- |
| Star | Each department LAN (hosts → access switch) | 2960-class access switch, end hosts | Cat6 UTP, T568B, RJ-45 | Easy fault isolation per host, simple to extend, industry-standard for LANs |
| Bus | Legacy comparison segment (lab demonstration only) | RG-58 coax, BNC T-connectors, 50Ω terminators | 10BASE2 coax | Included to contrast a shared collision-domain, single-point-of-failure design against the deployed star/mesh |
| Mesh (partial) | Router-to-router core (R-CORE, R-ADMIN, R-ACAD, R-ACCT) | Routers with serial/Gig interfaces, fibre or serial DTE/DCE links | Single-mode fibre / EIA-530 serial | Redundant paths so one link failure does not isolate a department – supports the failure-condition tests in Section 5 |
| Hybrid (overall) | Whole-organization network | Combination of the above | Mixed | Combines the low cost/simplicity of star access-layer wiring with the resilience of a meshed core – this is the topology actually implemented |

Device selection rationale:

- Access-layer switches (one per department) keep each LAN in its own broadcast domain and connect to the department router via a trunked or routed uplink.

- Serial/fibre WAN links (with clocking on the DCE side in the simulator) model the core mesh, since real inter-building or inter-floor backbone links are typically fibre.

- The bus segment is deliberately isolated (not carrying production traffic) and used only to visually and empirically compare collision-domain behaviour against the switched star segments.

# 4. Simulation Tool Usage & Validation

Recommended tool: Cisco Packet Tracer (readily available, supports routers, switches, servers, and a built-in traffic-generation/statistics view suited to this assignment). GNS3/EVE-NG are acceptable equivalents if closer-to-real IOS behaviour is needed.

## 4.1 Build & Verification Steps

- Place 4 routers (R-CORE, R-ADMIN, R-ACAD, R-ACCT), 4 switches, 33 end devices, and a server (Web/HTTP + DNS + FTP services enabled) per the topology in Section 3.

- Cable department hosts to their switch (Cat6 straight-through) and switches to routers; cable router-to-router core/mesh links as serial or Gig connections.

- Assign every interface the IPv4 address and mask from the VLSM table in Section 2.1; enable each router interface (no shutdown).

- Configure static routes (or OSPF) on all four routers as per Section 2.2; save configurations.

- On the server, configure DNS records (e.g. intranet.company.local) and start HTTP and FTP services.

- Verify with ping (ICMP) and tracert between at least one host per department and the server; confirm DNS resolution with nslookup; confirm HTTP access via a browser in each department.

- Capture command outputs (show ip route, show ip interface brief, ping/tracert results) as evidence in the Implementation & Results deliverable.

# 5. Protocol Analysis & Comparison (TCP, UDP, HTTP, DNS)

Behaviour is captured under three conditions: (A) Normal traffic – single client per test, (B) Increased traffic – multiple simultaneous client sessions generated toward the same server, and (C) Link congestion/failure – one core/mesh link saturated or shut down mid-test. Use Packet Tracer's Simulation mode / PDU statistics (or Wireshark captures in GNS3) to record the figures below; the rows are a template – replace the placeholder ranges with the actual measured values from your build before submission.

| Protocol | Condition A (Normal) | Condition B (Increased Load) | Condition C (Congestion/Failure) |
| --- | --- | --- | --- |
| TCP (file transfer) | Low latency, 0% loss, full throughput | Latency rises, throughput dips as windows/congestion control react, 0–2% loss (retransmitted) | Throughput drops sharply; retransmissions and possible timeouts; session recovers via rerouting if mesh backup exists |
| UDP (voice/video-style traffic) | Low latency, negligible loss | Latency rises modestly; some packet loss appears (no retransmission) | Higher loss/jitter, no automatic recovery – lost packets are simply dropped |
| HTTP (over TCP) | Fast page/object load | Noticeably slower page load under concurrent sessions | Page load stalls or times out until a route reconverges |
| DNS (UDP, occasionally TCP) | Near-instant resolution | Slightly longer resolution time under load | Resolution may fail/timeout and retry, or fail over to a secondary path if configured |

Expected engineering interpretation: TCP trades speed for reliability – its congestion control and retransmission keep data intact but reduce throughput and raise latency as conditions worsen. UDP keeps latency low but has no built-in recovery, so loss/jitter is the first visible symptom under load or failure, which is why it suits real-time traffic but not bulk transfer. HTTP's perceived performance simply inherits TCP's behaviour at the application layer. DNS is small in volume but latency-sensitive; under congestion its query/response can be delayed or dropped and retried, which is visible as a lookup delay before any application traffic even starts.

# 6. Performance Evaluation & Engineering Analysis

Suggested parameters to measure and record for each of the three conditions (fill in with actual simulator/PDU-statistics values):

| Metric | How to Measure in Packet Tracer | Normal | Increased Load | Congestion/Failure |
| --- | --- | --- | --- | --- |
| Latency (ms) | Simulation mode PDU delay, or extended ping round-trip time | – | – | – |
| Throughput | File/FTP transfer size ÷ completion time shown in PDU list | – | – | – |
| Packet loss (%) | Sent vs. successful PDUs in Simulation Panel | – | – | – |
| Successful delivery rate | Ratio of “successful” to total PDUs, realtime mode | – | – | – |

Analysis should plot these (e.g. a line chart of latency across the three conditions per protocol, and a bar chart of packet loss per protocol) and discuss: which protocol degraded first and why; whether the mesh backup link masked the failure for TCP/HTTP sessions but not for a UDP stream in progress; and where the bottleneck was – access-layer switch, core link bandwidth, or server processing.

Identified limitations of the design (to discuss with real data once captured):

- A single server hosting DNS, HTTP and FTP is a shared bottleneck and single point of failure for services (though not for routing, since the network path itself is meshed).

- VLSM leaves limited headroom in the smaller /28 department subnets (14 usable addresses) if departments grow beyond current staffing.

- Static routing does not automatically adapt to new links being added; OSPF would remove this maintenance burden at the cost of slightly more CPU/control-plane overhead on routers.

# 7. Broader Considerations & Professional Responsibility

- Scalability: each department subnet retains spare addresses beyond current headcount; the /24 block itself leaves room to add further /28-/29 subnets for future departments.

- Reliability: the meshed core means a single inter-router link failure does not isolate any department, directly supporting the Section 5(C) failure tests.

- Standards compliance: cabling follows TIA/EIA-568 (Cat6 UTP, T568B), Ethernet framing follows IEEE 802.3, and addressing follows RFC 1918 private address space with VLSM per RFC 1878.

- Security & safe configuration practice: enable password/secret on all router lines, disable unused switch ports, and consider VLAN separation per department in addition to Layer-3 subnetting.

- Resource efficiency (SDG 9): VLSM avoids wasting address space, and the hybrid topology reuses existing star-wired LANs rather than requiring a full mesh of expensive links to every host, aligning with efficient-infrastructure goals.

- Documentation: all device configurations should be saved and version-controlled (see Section 9) so the design is reproducible and auditable.

# 8. Pseudocode Deliverable

## 8.1 VLSM Subnet Allocation Algorithm

BEGIN VLSM_Allocate(baseNetwork, prefixLength, departmentList)

SORT departmentList BY hostsRequired DESCENDING

currentAddress = baseNetwork

FOR EACH department IN departmentList

requiredHosts = department.hostsRequired

neededPrefix = 32 - CEIL(LOG2(requiredHosts + 2))   // +2 for network & broadcast

subnetSize = 2 ^ (32 - neededPrefix)

department.subnet = currentAddress

department.mask = neededPrefix

department.gateway = currentAddress + 1

department.broadcast = currentAddress + subnetSize - 1

currentAddress = currentAddress + subnetSize

END FOR

RETURN departmentList   // each entry now has subnet, mask, gateway, broadcast

END

## 8.2 Basic Static-Route Configuration Logic

BEGIN Configure_Router(router, interfaceList, routeTable)

FOR EACH iface IN interfaceList

SET iface.ipAddress, iface.subnetMask

ENABLE iface

END FOR

FOR EACH route IN routeTable

ADD static_route(route.destinationNetwork, route.mask, route.nextHop)

END FOR

IF router.role == "department-edge" THEN

ADD default_route(0.0.0.0, 0.0.0.0, router.coreFacingInterface)

END IF

SAVE router.configuration

END

## 8.3 Protocol Performance Test Driver

BEGIN Run_Performance_Tests(protocolList, conditionList)

FOR EACH condition IN conditionList          // Normal, Increased, Congestion/Failure

APPLY_CONDITION(condition)                 // e.g. add background traffic, shut a link

FOR EACH protocol IN protocolList           // TCP, UDP, HTTP, DNS

startTime = NOW()

result = SEND_TEST_TRAFFIC(protocol)

endTime = NOW()

RECORD(protocol, condition, {

latency: endTime - startTime,

throughput: result.bytesTransferred / (endTime - startTime),

packetLoss: 1 - (result.received / result.sent)

})

END FOR

REVERT_CONDITION(condition)

END FOR

EXPORT results_table

END

# 9. GitHub Upload

Create a repository (e.g. computer-networks-assignment) and upload:

- The Packet Tracer / GNS3 project file (.pkt / .gns3project).

- This report (PDF/DOCX) and any exported PDU-statistics / captured screenshots as an evidence folder.

- A README.md summarising the topology, VLSM plan, and how to reproduce the three test conditions.

- A commit history that reflects incremental progress (design → configuration → testing → report), which itself serves as part of the “Simulation Tool Usage & Validation” evidence.

# 10. Reflection (Template – personalize before submission)

The paragraph below is a starting template only. Replace it with your own 300–500 word account of what you actually built, what went wrong, and how you fixed it – graders can tell a generic reflection from a genuine one.

Working through this assignment gave me a much clearer picture of how addressing, topology and protocol behaviour interact in practice rather than as separate textbook topics. Deriving the VLSM plan forced me to think about actual headcounts per department instead of picking a subnet size arbitrarily, and I had to re-check my broadcast/gateway addresses twice after an off-by-one error left one department's gateway inside another subnet's range. Building the physical topology in the simulator was straightforward for the star segments, but getting the core mesh links to come up required double-checking clock rate settings on the DCE side of the serial links – something I had not appreciated mattered until interfaces stayed in a down/down state. The most useful troubleshooting step was working interface-by-interface with show ip interface brief rather than assuming the whole design was wrong. Running the protocol comparisons was the part that tied the exercise together: seeing TCP file transfers slow down but still complete under a shut core link, while a UDP stream in progress simply lost data with no recovery, made the reliability-versus-latency trade-off concrete in a way that reading about it never had. It also showed me the practical value of the mesh backup path – without it, a single link failure would have isolated an entire department, not just degraded its performance. If I extended this project, I would replace the static routes with OSPF to reduce ongoing configuration effort, and I would separate DNS/HTTP/FTP onto distinct virtual servers to remove the single shared bottleneck identified in Section 6. Overall, the exercise connected the addressing, topology and protocol-analysis pieces of the course in a way that a single-topic lab would not have.