## 1. Debugging IPv4 Workstation Configuration

Given configuration:

IP address: 192.168.10.65
Subnet mask: 255.255.255.192
Prefix length: /26
Default gateway: 192.168.10.1

The block size for /26 is:

256-192=64

The /26 subnet ranges are:

- 192.168.10.0 to 192.168.10.63

- 192.168.10.64 to 192.168.10.127

- 192.168.10.128 to 192.168.10.191

- 192.168.10.192 to 192.168.10.255

The IP address 192.168.10.65 belongs to:

192.168.10.64/26

Network address:

Valid host range:

Broadcast address:

The configured IP address 192.168.10.65 is valid in the subnet 192.168.10.64/26.

However, the gateway 192.168.10.1 belongs to the subnet 192.168.10.0/26. Therefore, the IP address and gateway are in different subnets.

The configuration error is the incorrect default gateway.

To communicate with systems in 192.168.10.0/26, the workstation can be configured as:

IP address: 192.168.10.2
Subnet mask: 255.255.255.192
Default gateway: 192.168.10.1

Corrected subnet details:

Network address: 192.168.10.0
Valid host range: 192.168.10.1 to 192.168.10.62
Broadcast address: 192.168.10.63

Usable host addresses:

2^{32-26}-2

2^6-2=62

Therefore, the corrected subnet supports

## 2. Debugging IPv6 Device Configuration

Given addresses:

Device A:

2001:db8::ff00:42:8329/64

Device B:
2001:db8:0:0:1::1/64

### Device A Expanded Address

Network prefix:

### Device B Expanded Address

Network prefix:

Both devices have the same first 64 bits. Therefore, they belong to the same subnet.

The communication failure is not caused by the /64 network prefix. It may be caused by incorrect interface configuration, duplicate addresses, disabled IPv6, firewall restrictions or incorrect router settings.

Suitable corrected addresses are:

Corrected network prefix:

Number of host bits:

128-64=64

Total possible host addresses:
2^{64}

## 3. Debugging and Optimising the Subnet Allocation

Given network:

192.168.1.0/24

The departments use /26, /27 and /28 subnet masks.

### /26 Subnet

Subnet mask:

255.255.255.192

Total addresses:

2^{6}=64

Usable addresses:

64-2=62

Network address:

192.168.1.0

Valid host range:

192.168.1.1192.168.1.62

Broadcast address:

192.168.1.63

### /27 Subnet

Subnet mask:

255.255.255.224

Total addresses:

2^{5} =32

Usable addresses:

32-2=30

Network address:

192.168.1.64

Valid host range:

192.168.1.65192.168.1.94

Broadcast address:

192.168.1.95

### /28 Subnet

Subnet mask:

255.255.255.240

Total addresses:

2^{4} =16

Usable addresses:

16-2=14

Network address:

192.168.1.96

Valid host range:

192.168.1.97192.168.1.110

Broadcast address:

192.168.1.111

| Subnet | Network Address | Valid Host Range | Broadcast Address | Usable Hosts |
| --- | --- | --- | --- | --- |
| /26 | 192.168.1.0 | 192.168.1.1–192.168.1.62 | 192.168.1.63 | 62 |
| /27 | 192.168.1.64 | 192.168.1.65–192.168.1.94 | 192.168.1.95 | 30 |
| /28 | 192.168.1.96 | 192.168.1.97–192.168.1.110 | 192.168.1.111 | 14 |

Total allocated addresses:

64+32+16=112

Remaining unused addresses:

256-112=144

Unused address range:

A /28 subnet is insufficient for a department requiring more than 14 hosts. A /26 subnet is inefficient when only a small number of devices are present.

Subnet allocation mistakes include overlapping subnet ranges, assigning a host address as a network or broadcast address and selecting subnet masks without considering the required number of hosts.

An optimized VLSM allocation is:

| Department Requirement | Allocated Subnet | Host Range | Broadcast | Usable Hosts |
| --- | --- | --- | --- | --- |
| Up to 60 hosts | 192.168.1.0/26 | 192.168.1.1–192.168.1.62 | 192.168.1.63 | 62 |
| Up to 30 hosts | 192.168.1.64/27 | 192.168.1.65–192.168.1.94 | 192.168.1.95 | 30 |
| Up to 14 hosts | 192.168.1.96/28 | 192.168.1.97–192.168.1.110 | 192.168.1.111 | 14 |
| Up to 14 hosts | 192.168.1.112/28 | 192.168.1.113–192.168.1.126 | 192.168.1.127 | 14 |

The remaining range after optimization is:

VLSM improves address utilization by assigning larger subnets to departments with more devices and smaller subnets to departments with fewer devices.

## 4. Debugging IPv6 Device Configuration

Given addresses:

Device A:

2001:db8::ff00:42:8329/64

Device B:

2001:db8:0:0:1::1/64

### Device A Expanded Address

Network prefix:

### Device B Expanded Address

Network prefix:

Both addresses have the same first four hexadecimal groups. Therefore, both devices belong to the same /64 subnet.

There is no network-prefix mismatch. The communication failure may be caused by firewall settings, duplicate IP addresses, disabled IPv6 interfaces or incorrect network configuration.

Suitable optimized addresses are:

Device A:

Device B

Corrected network prefix:

Number of host addresses:

2{128-64}=2{64}

## 5. Debugging the Routing Table

Given routing table:

| Destination Network | Next Hop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| Default route | 10.0.0.1 |

Destination IP address:

192.168.2.100

The subnet mask for /24 is:

255.255.255.0

The destination address belongs to the subnet:

The matching routing entry using the longest-prefix match is:

The selected next-hop router is:

The valid host range of the destination subnet is:

192.168.2.1192.168.2.254

The broadcast address is:

192.168.2.255

Therefore, 192.168.2.100 is a valid host address in the intended subnet.

If packets are not reaching the destination, possible errors include an unreachable next-hop router, incorrect interface configuration, missing return route, firewall blocking or an incorrect next-hop address.

The optimized routing table is:

| Destination Network | Next Hop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| 0.0.0.0/0 | 10.0.0.1 |

The router should verify that 10.0.0.3 is reachable through a directly connected interface.

If no specific destination matches, the router uses: